PowerMind AI – Technical Architecture Breakdown

🔹 1. Monitoring (Sensing + Data Collection)

What it does: Collects raw data from electricity flow in real time.

✅ Key Concepts:

- Current sensors (ACS712): detect amperage usage

- Voltage dividers: measure voltage if needed (optional for AC safety)

- MCP3008 ADC: converts analog sensor data into digital for Raspberry Pi

- Sampling frequency: How often data is collected (e.g. every second)

- CSV + Cloud Logging: store every measurement with a timestamp

🧠 What AI learns from here:

- Patterns of usage over time

- Abnormal spikes

- Idle consumption

- Appliance-level signatures (later phase)

🔹 2. Optimization (Data Analysis + Energy Efficiency AI)

What it does: Analyzes past and real-time data to make smarter decisions.

✅ Key Concepts:

- Python-based logic or ML models for rule-based decisions

- Time-of-day usage prediction (e.g., when peak load occurs)

- Energy efficiency scoring: room/device/wattage prioritization

- Reinforcement Learning (future): AI learns from reward/punishment (e.g., minimize wattage without loss of comfort)

Example:

“At 6pm daily, the water heater and fridge overlap usage — delay heater by 20 min to avoid peak surge”

🔹 3. Control (Relay Switching + Automation)

What it does: Actually turns devices on/off based on smart decisions.

✅ Key Concepts:

- GPIO control of relays (Python: GPIO.output(pin, HIGH/LOW))

- Emergency cut-off for high-load conditions

- Priority list: essential vs. optional devices

- Manual override: user can always switch via web/mobile

Safety Layer:

- Use optocouplers or isolation circuits for relay safety

- Add thermal shutdown logic if devices overheat

🔹 4. Prediction & Resistance Management (AI Models)

What it does: Predicts resistance drops, power loss, or overconsumption trends.

✅ Key Concepts:

- Ohm’s Law (V = IR) used to estimate resistance

- Power loss detection (resistive heat = I²R)

- Anomaly detection models (unsupervised ML or LSTM)

- AI flags:

  - Corroded wires (increased resistance)

  - Unexpected drops

  - Ghost current draw

📊 Sample Architecture Flow

Sensors (ACS712 → MCP3008)

↓

Raspberry Pi (Data logger + Real-time processing)

↓

Flask API + Web Dashboard

↓

Control Decisions (Auto-switch via Relay)

↓

Cloud Sync + AI Learning

🔹 1. Monitoring Layer – The Nervous System

Purpose: Capture all electrical activity (current, voltage, resistance, flow) from the system in real time.

🚧 Components:

- ACS712/ACS758 → measures current flow

- Voltage sensors (optional) for AC/DC voltage

- MCP3008 ADC → converts analog signal to digital

- Raspberry Pi GPIO/ADC interface

🧠 Output:

- Time-series data of amperage, optionally voltage, and calculated power

- Data logged locally to .csv or streamed to dashboard

- Raw input for future AI analysis

- 🔹 2. Optimization Layer – The Brain

Purpose: Analyze electricity usage patterns and make smart efficiency decisions.

🚧 Technologies:

- Python logic + basic heuristics (rules: “if >10A, cut load”)

- Optionally: AI models (MLP, Decision Trees, or Edge Impulse AI)

🧠 Key Functions:

- Detect high-consumption periods

- Predict peak usage based on time-of-day trends

- Recommend or auto-optimize:

  - When to delay water heater

  - When solar is strong → prioritize heavy loads

  - Turn off idle devices intelligently

🔹 3. Control Layer – The Hands

Purpose: Execute decisions by turning devices on/off, rerouting flow, or isolating fault circuits.

🚧 Components:

- Relay modules (1–8 channel, 5V or 12V) (up to which channel do we need?)

- Raspberry Pi controls GPIO to activate relays

- Power routing through smart sockets or contactors (do we need to add this smart sockets or contactors)

🧠 Key Features:

- Auto-cut on overload

- Manual override via dashboard/mobile

- Priority control:

  - Essentials (lights/fridge) always on

  - Non-essentials (AC/charger) switched off if needed

🔹 4. Intelligence Layer – The Soul

Purpose: Long-term learning and prediction based on all historical usage.

🚧 Tools:

- AI platforms: TensorFlow Lite, Edge Impulse, or custom ML in Python (which one we should use)

- Data: Stored in cloud or local DB (MongoDB/SQLite) (which one we should use)

- Algorithms:

  - Anomaly detection

  - Reinforcement learning

  - Time-series prediction (LSTM)

🧠 Learning Goals:

- Predict next 24h usage

- Identify energy leaks or abnormal usage

- Recommend load shifting (e.g., night-time laundry)

- Build a smart profile for each household

🧩 How All Layers Connect (Simple Data Flow)

mathematica

Copy

Sensors  →  Pi + Logic  →  Dashboard/API  →  AI Model (Cloud or Edge)

↓              ↓               ↓                ↓

Real-time      Decision      Control       Long-term Prediction

- 🔍 Step 1: Overview of the Monitoring Layer

🎯 Goal:

- Capture real-time data on:

  - Current (Amps)

  - Voltage (Volts)

  - Power (Watts) → calculated from Voltage × Current

📦 Main Hardware:

| Sensor | Measures | Output Type |
| --- | --- | --- |
| ACS712 / ACS758 | Current | Analog Voltage (0–5V) |
| 0–25V Voltage Module | Voltage (DC) | Analog Voltage (scaled down) |
| MCP3008 | ADC (Analog to Digital) | SPI to Raspberry Pi |

⚙️ Step 2: Wiring Plan

🔌 Current Sensor (ACS712 or ACS758):

- VCC → 5V on Raspberry Pi

- GND → Ground on Raspberry Pi

- OUT → MCP3008 Channel 0 (CH0)

🔌 Voltage Sensor (0–25V Module):

- VCC → 5V on Raspberry Pi

- GND → Ground

- OUT → MCP3008 Channel 1 (CH1)

🔌 MCP3008 ADC:

- CH0 → from ACS712 OUT

- CH1 → from Voltage Sensor OUT

- VDD, VREF → 3.3V

- AGND, DGND → GND

- CLK, DOUT, DIN, CS → GPIO pins (we’ll wire them in next step)

🔧 How It Works:

- Reads voltage from CH1 (0–25V sensor)

- Reads current from CH0 (ACS712 30A)

- Calculates power (Watts)

- Prints live status in terminal

- Logs data to power_log.csv

Top of Form

Bottom of Form